COACH M8

Hardware Sensor Stack

Technical Specification & Bill of Materials

v1.0

Prepared for: The Maker Football Incubator

Date: March 24, 2026

Classification: Confidential

# 1. Executive Summary

This document specifies the complete hardware sensor stack for Coach M8, an AI-powered football performance analysis platform. The stack replaces industry-standard Catapult Vector ($500–$1,500/athlete) + WHOOP ($360/yr/athlete) with an open-architecture system at approximately $128–$135 per athlete, delivering comparable or superior raw data access.

The system consists of three wearable devices per athlete plus pitch-side infrastructure, providing real-time ECG-grade heart rate, 9-axis motion tracking, GPS positioning, BLE 5.1 direction-finding, and 24/7 recovery monitoring — all streaming live into the Coach M8 platform.

# 2. System Architecture Overview

## 2.1 Per-Athlete Wearable Stack

| Device | Module | Worn | Function | Cost |
| --- | --- | --- | --- | --- |
| HR Sensor | Movesense HR2 | Chest strap | ECG, HR, IMU, Temp | ~$95 |
| Position Tag | RF-BM-2652P1 + GP-02 | Vest pocket | GPS, BLE 5.1 AoA | ~$8 |
| Recovery Band | Realtek RTL8762E | Wrist (24/7) | Sleep, SpO2, RHR | ~$25 |

Total per athlete: $128–$135

## 2.2 Pitch-Side Infrastructure

| Component | Module | Quantity | Cost |
| --- | --- | --- | --- |
| BLE Gateway | Minew G1 or 3× ESP32-S3 | 1–3 units | $15–$80 |
| Edge Server | Raspberry Pi 4 (4GB) | 1 unit | ~$75 |
| AoA Anchors (optional) | RF-BM-2652P1 on tripods | 4–6 units | $200–$400 ea |
| AI Camera | MatchMate Q1 | 1 unit | $800–$1,200 |

Total infrastructure (without AoA): ~$1,000–$1,400

Total infrastructure (with AoA): ~$2,500–$4,000

## 2.3 Cost Comparison

|  | Coach M8 Stack | Catapult + WHOOP |
| --- | --- | --- |
| Per athlete (hardware) | $128–$135 | $500–$1,500 |
| Per athlete (annual) | $0 (no subscription) | $360/yr (WHOOP) |
| Infrastructure (one-time) | $1,000–$4,000 | $50,000–$150,000+ (LPS) |
| 30 athletes + infra (Year 1) | $5,000–$8,000 | $75,000–$200,000+ |
| Raw data access | Full (open API/SDK) | Limited (proprietary) |

# 3. Device 1: Movesense HR2 (Chest Sensor)

The Movesense HR2 is a Finnish-designed medical-grade sensor built on the Nordic nRF52832 SoC. It provides ECG-grade heart rate monitoring and full 9-axis inertial measurement — the core physiological and biomechanical data layer.

## 3.1 Specifications

| Manufacturer | Movesense (Suunto/Amer Sports), Finland |
| --- | --- |
| SoC | Nordic nRF52832 — ARM Cortex-M4F, 64 MHz, 512KB Flash, 64KB RAM |
| BLE | Bluetooth 4.2 Low Energy |
| ECG | Single-channel, 125–512 Hz sampling, raw waveform output (0.38 µV resolution), configurable filters (LP: 40/100/150 Hz, HP: 0.5 Hz) |
| Heart Rate | Derived from ECG, ±5 bpm accuracy, real-time RR intervals (ms) |
| HRV | RMSSD, SDNN derivable from raw RR intervals in real-time |
| Accelerometer | 3-axis, 13–1666 Hz sampling, ±2g to ±16g range |
| Gyroscope | 3-axis, 13–1666 Hz sampling, 245–2000 dps range |
| Magnetometer | 3-axis, 13–1666 Hz sampling |
| Temperature | Internal sensor, 233–398 K, ±1 K accuracy |
| Memory | 384 KB EEPROM (offline data logging) |
| Battery | CR2025 coin cell, ~1 year standby, 10–16 hrs active streaming |
| Size / Weight | 36.5 mm diameter, 10.4 mm thick, ~10g |
| Water Resistance | IP67+ (sweat, rain, washable strap) |
| Certifications | CE, medical-grade variant available (MDR Class IIa) |
| SDK | Open REST API, C++ custom firmware, Android/iOS SDKs, Python/WebBLE clients |
| Price | €89 per unit (~$95 USD) at 100+ quantity |

## 3.2 API Endpoints Used by Coach M8

| Endpoint | Data | Rate | Coach M8 Field |
| --- | --- | --- | --- |
| /Meas/HR | HR bpm + RR intervals | Real-time | hr_avg, hr_max, hr_zones |
| /Meas/ECG/{Hz} | Raw ECG waveform | 125–512 Hz | HRV, cardiac screening |
| /Meas/IMU9 | Acc + Gyro + Mag | Up to 1666 Hz | Player Load, accel/decel |
| /Meas/Temp | Skin temperature | On change | Heat stress monitoring |

## 3.3 Derived Metrics (Computed on Edge Server)

| Metric | Formula / Method | Replaces |
| --- | --- | --- |
| Player Load | √(Δax² + Δay² + Δaz²) accumulated, scaled to ~1000 | Catapult PlayerLoad |
| TRIMP | Duration × HR fraction × e^(1.92 × HR fraction) — Banister method | Catapult TRIMP |
| HR Zone % | Time in each zone / total time × 100 (5 zones) | Catapult/WHOOP zones |
| HR Recovery 60s | HR at exercise end − HR at 60 seconds post | Custom metric |
| HRV RMSSD | √(mean of squared successive RR differences) | WHOOP HRV |
| Accel Events | Count of acc magnitude > 3 m/s² threshold | Catapult accel count |
| Decel Events | Count of decel magnitude < −3 m/s² threshold | Catapult decel count |

# 4. Device 2: Position Tag (GPS + BLE 5.1)

A custom-assembled tag combining the RF-Star RF-BM-2652P1 BLE 5.1 module with the Ai-Thinker GP-02 GPS module. Provides outdoor GPS positioning at 10Hz and BLE 5.1 direction-finding for optional indoor AoA positioning.

## 4.1 RF-BM-2652P1 (BLE 5.1 Module)

| Manufacturer | Shenzhen RF-Star Technology (China) |
| --- | --- |
| Part Number | RF-BM-2652P1 |
| SoC | Texas Instruments CC2652P — 48 MHz ARM Cortex-M4F |
| Protocols | BLE 5.1, Thread, Zigbee 3.0, IEEE 802.15.4, 6LoWPAN |
| BLE 5.1 Features | LE 1M/2M PHY, Coded PHY (long range), Direction Finding (AoA/AoD), Bluetooth Mesh |
| TX Power | +20 dBm (100mW) |
| BLE Range | 500m (open air) — covers full football pitch with margin |
| GPIO | 23 pins (2 used for UART to GPS, 21 available) |
| Package | 25.0 × 16.4 × 2.2 mm, 30-pin half-hole |
| Certifications | CE, FCC (ID: 2ABN2-BM2652P1) — pre-certified, no lab testing needed |
| Price | ~$4–$6 per unit |

## 4.2 GP-02-KIT (GPS Module)

| Manufacturer | Ai-Thinker (China) |
| --- | --- |
| Chipset | AT6558R — BDS/GNSS multi-mode satellite navigation SoC |
| Constellations | GPS + BeiDou + GLONASS + Galileo + QZSS + SBAS (6 systems simultaneously) |
| Update Rate | 10 Hz (matches Catapult standard) |
| Position Accuracy | 2.0–2.5m CEP (open sky) |
| Cold Start | ~30 seconds |
| Hot Start | <2 seconds |
| Dead Reckoning | Supported (hybrid navigation with IMU data) |
| Interface | UART (NMEA 0183), default 9600 baud, configurable to 256,000 bps |
| Processor | 32-bit ARM, 133 MHz — headroom for edge computing |
| Size | 10.3 × 9.9 × 2.4 mm (smallest in class) |
| Voltage | 2.7V–3.6V (direct coin cell compatible) |
| Temp Range | −40°C to 85°C (covers Cairo summer conditions) |
| Price | ~$2–$3 per unit |

## 4.3 Combined Tag Assembly

Total BOM cost: ~$8 per tag (module + GPS + CR2032 holder + PCB)

Dimensions: ~30mm × 20mm × 8mm (fits in vest pocket)

Power: CR2032 coin cell (GPS draws ~30mW — estimated 6–8 hrs active tracking)

UART wiring: GP-02 TX → CC2652P RX, GP-02 RX → CC2652P TX, shared GND + 3.3V VCC

Firmware: CC2652P parses NMEA, packs lat/lon/speed/heading into BLE advertising payload + CTE for AoA

## 4.4 Positioning Accuracy

| Mode | Accuracy | Anchors Needed | Use Case |
| --- | --- | --- | --- |
| GPS outdoor | 2.0–2.5m | None (satellites) | Main pitch (default) |
| BLE 5.1 AoA indoor | 0.5–1.0m | 4–6 anchors | Indoor gym (optional) |
| GPS + AoA fusion | 1.0–2.0m | 4–6 anchors | Covered areas, stadium |
| Catapult (GPS mode) | 1.5–2.0m | None | Industry standard |
| KINEXON (UWB LPS) | 8–9 cm | 12+ anchors | FIFA/Bundesliga premium |

## 4.5 Live Position Metrics Derived

| Metric | Source | Coach M8 Field |
| --- | --- | --- |
| Position (x, y) | GPS lat/lon → pitch coordinates via 4-corner calibration | player_positions |
| Total distance | Sum of position deltas over session | total_distance_m |
| Speed / Max speed | GPS ground speed (NMEA $GPRMC) or position delta / time | max_speed_kmh |
| Sprint count | Count of speed > 25 km/h sustained > 1 second | sprint_count |
| High-speed running | Distance accumulated at speed > 19.8 km/h | cv_metrics |
| Heatmap | Time spent in each 5×5m pitch grid zone | Dashboard viz |
| Formation shape | Average (x,y) of all players in each position group | tactical_metrics |

# 5. Device 3: Realtek Recovery Wristband (24/7)

An OEM smart wristband based on the Realtek RTL8762E chipset, worn 24/7 for off-pitch recovery and readiness monitoring. Replaces WHOOP’s core functionality without the $30/month subscription.

| Chipset | Realtek RTL8762E — ARM Cortex-M4F, BLE 5.2 |
| --- | --- |
| HR Sensor | PPG (optical) — continuous 24/7 heart rate monitoring |
| SpO2 | Blood oxygen via red/IR LED PPG |
| Sleep Tracking | Light / Deep / REM / Awake staging (accelerometer + PPG algorithm) |
| HRV | Derived from PPG inter-beat intervals (resting/sleep measurement) |
| Skin Temperature | Available on some OEM models |
| Activity | Step count, calories, active minutes |
| Battery | 5–7 days typical (varies by OEM) |
| Water Resistance | IP68 (swimming/shower safe) |
| SDK | OEM-dependent — request SDK documentation with sample order |
| Price | ~$15–$40 per unit (OEM, volume-dependent) |

## 5.1 Recovery Metrics Derived (Coach M8 Algorithm)

| Metric | Inputs | Replaces |
| --- | --- | --- |
| Recovery Score (0–100%) | RHR trend + HRV + sleep quality + SpO2 + skin temp | WHOOP Recovery |
| Strain Score (0–21) | Cumulative HR zone time + daily activity intensity | WHOOP Strain |
| Sleep Score (0–100%) | Sleep duration vs. need + efficiency + stage balance | WHOOP Sleep |
| Readiness Flag | Recovery + sleep + ACWR combined → Green/Yellow/Red | Coach M8 unique |

Note: These are Coach M8 proprietary algorithms, not black-box scores from a third party. They can be tuned for youth athletes, Cairo’s heat conditions, Ramadan fasting periods, and The Maker’s specific training load patterns.

# 6. AI Camera: MatchMate Q1

The MatchMate Q1 is a Chinese-manufactured AI auto-tracking panoramic camera for football match and training recording, providing live streaming, auto-highlights, and computer vision data.

| Resolution | Panoramic: 7168×2160, AI Tracking: 1920×1080 |
| --- | --- |
| Frame Rate | 50 fps |
| Imaging | 16 MP CMOS, f/2.0 aperture |
| FOV | Horizontal: 200°, Vertical: 60° (covers full 11-a-side pitch) |
| Live Streaming | RTMP + SRT via Wi-Fi 6 or Ethernet |
| Connectivity | Wi-Fi 6, 4G LTE (TDD/FDD), Bluetooth, Ethernet |
| AI Features | Auto-tracking (football, basketball, hockey, rugby), instant replay, auto-highlights |
| Storage | 100 GB internal (~50 hrs footage) |
| Battery | 10 hrs (5 hrs at max load) |
| Weather | IP65+ (heavy rain resistant) |
| Mount | Standard 1/4”-20 tripod thread |
| Dimensions | 5 cm thick × 20 cm long |
| Origin | Zhejiang, China |
| Price | ~$800–$1,200 (estimated, inquiry pending) |

## 6.1 Integration with Coach M8

- SRT stream over Ethernet → FFmpeg on Raspberry Pi → HLS/WebRTC to /live dashboard

- Video clips synced with wearable timestamps for the video_tags table

- AI tracking output provides secondary position verification against GPS data

- Pending confirmation: SDK/API for programmatic control + RTSP + tracking data export

# 7. Live Data Pipeline

All sensor data flows through a pitch-side edge server (Raspberry Pi 4) which performs sensor fusion, metric computation, and real-time upload to Supabase.

## 7.1 Data Flow

- Movesense HR2 → BLE 4.2 → Gateway → Pi (HR, ECG, IMU streams)

- Position Tag → BLE 5.1 advertising → Gateway → Pi (GPS lat/lon/speed + AoA position)

- Pi Fusion Engine → matches player_id, timestamp-aligns (±10ms), computes derived metrics

- Pi → Supabase → HTTPS POST batched inserts every 1 second (10 readings/player/batch)

- Supabase Realtime → WebSocket → Coach M8 /live dashboard (coaches’ iPads)

## 7.2 Data Volume

- Position data: 10 Hz × 30 players = 300 rows/sec = ~1.6M rows per 90-min session

- HR data: 1 Hz × 30 players = 30 rows/sec = ~162K rows per session

- IMU data: processed on-edge, only derived metrics sent (Player Load per minute)

- Supabase insert batching: 300 rows/sec is well within Supabase’s capacity

## 7.3 End-to-End Latency

| Step | Latency |
| --- | --- |
| GPS fix (10Hz) | 100 ms |
| UART → CC2652P parse | ~5 ms |
| BLE broadcast → gateway | ~10 ms |
| Gateway → Pi (WiFi) | ~5 ms |
| Pi → Supabase (HTTPS) | ~50–150 ms |
| Supabase Realtime → browser | ~50–100 ms |
| TOTAL END-TO-END | ~250–400 ms |

# 8. Complete Bill of Materials (30 Players)

## 8.1 Wearables (30 Athletes)

| Item | Part | Qty | Unit $ | Total $ | Source |
| --- | --- | --- | --- | --- | --- |
| Movesense HR2 | HR2 sensor | 30 | $95 | $2,850 | movesense.com |
| RF-BM-2652P1 | CC2652P mod | 30 | $5 | $150 | RF-Star |
| GP-02-KIT | AT6558R GPS | 30 | $2 | $60 | Ai-Thinker |
| Tag PCB + battery | Custom PCB | 30 | $1 | $30 | JLCPCB |
| CR2032 batteries | Coin cell | 60 | $0.30 | $18 | Amazon/local |
| Realtek wristband | RTL8762E OEM | 30 | $25 | $750 | Alibaba OEM |
| Chest straps | Textile strap | 30 | $5 | $150 | Movesense/OEM |
| Training vests | With tag pocket | 30 | $8 | $240 | Luo Da / Alibaba |

Wearables subtotal: $4,248

## 8.2 Infrastructure (One-Time)

| Item | Qty | Unit $ | Total $ | Source |
| --- | --- | --- | --- | --- |
| ESP32-S3 gateways | 3 | $5 | $15 | Espressif / AliExpress |
| Raspberry Pi 4 (4GB) | 1 | $75 | $75 | Local supplier |
| MatchMate Q1 camera | 1 | $1,000 | $1,000 | Alibaba |
| Camera tripod + Ethernet | 1 | $50 | $50 | Local |
| AoA anchors (optional) | 6 | $300 | $1,800 | RF-Star / custom |

Infrastructure subtotal (without AoA): $1,140

Infrastructure subtotal (with AoA): $2,940

| GRAND TOTAL (30 players, no AoA) | $5,388 |
| --- | --- |
| GRAND TOTAL (30 players, with AoA) | $7,188 |
| Equivalent Catapult + WHOOP (30 players, Year 1) | $75,000–$200,000+ |

# 9. Development Roadmap

## Phase 1: Proof of Concept (Weeks 1–4)

- Order 5× Movesense HR2 samples (€445)

- Order 5× GP-02-KIT + 5× RF-BM-2652P1 dev boards (~$50)

- Wire GP-02 UART → CC2652P, flash Nordic/TI peripheral_uart sample

- Build Raspberry Pi gateway: connect to Movesense + tags, push to Supabase

- Validate end-to-end: live dots on pitch diagram in Coach M8 /live page

## Phase 2: Field Testing at The Maker (Weeks 5–8)

- Order 30× full kits + 30× Realtek wristband samples

- Calibrate GPS-to-pitch coordinate mapping at The Maker’s pitch in Cairo

- Run 2-week parallel test vs. manual tracking / coach observation

- Tune Player Load algorithm for youth body mass normalization

- Build recovery score algorithm using wristband sleep + HRV data

- Order and test MatchMate Q1 camera with SRT → /live integration

## Phase 3: Production Deployment (Weeks 9–12)

- Design production tag PCB (RF-BM-2652P1 + GP-02, JLCPCB fabrication)

- Order production vests from Luo Da with integrated tag pockets

- Deploy full 80-player system across all age groups at The Maker

- Launch live dashboard for coaches and weekly AI reports for parents

## Phase 4: Proprietary Hardware (Months 4–6, Optional)

- Design single combined PCB: CC2652P + GP-02 + LSM6DSO IMU + MAX30003 ECG AFE

- Eliminates Movesense dependency, drops per-athlete cost to ~$25–$30

- Full Coach M8 proprietary sensor pod — hardware moat

This specification is based on research conducted March 2026. All pricing should be confirmed with suppliers before procurement. Technical specifications should be validated against manufacturer datasheets.